# Introduction to IT (tutorial)

# Step 1: Understand and Define the Problem (Analyse)

- The problem is to design a combinational logic circuit for a car seatbelt alarm system. The system should turn on the car alarm either the driver seat or the passenger seat is occupied, but the seat belt is not fastened.

- The inputs to the system are DRIV and PASS, which indicate whether the driver or passenger seat is occupied.

- IGN, which shows that the car ignition is on.

- BELT_D̅ and BELT_P̅, which indicate that the driver’s and passenger’s seatbelts are unfastened (active-LOW signals).

- The output of the system is ALARM, which is also active-LOW, meaning the alarm is activated when the output is 0 and off when the output is 1.

- The circuit must ensure that the alarm only activates when the ignition is on and a seated occupant has not fastened their seatbelt.

# Step 2: Organise and Describe the Data

Inputs:

DRIV = 1 (when the driver seat is occupied)

DRIV = 0 (when the driver seat is not occupied)

PASS = 1 when the passenger seat is occupied

PASS = 1 when the passenger seat is not occupied

IGN = 1 when the car ignition is ON

IGN = 0 when the car ignition is OFF

BELT_D̅ = 0 when the driver seatbelt is unfastened

BELT_D̅ = 1 when the driver seatbelt is fastened

BELT_P̅ = 0 when the passenger seatbelt is unfastened

BELT_P̅ = 1 when the passenger seatbelt is fastened

Output:

ALARM (active-LOW)

0 → Alarm ON

1 → Alarm OFF

# Step 3: Plan the Solution (Design the Algorithm)

3.1 Algorithm (in plain English)

Step 1: Read inputs: DRIV, PASS, IGN, BELT_D̅, BELT_P

Step 2: Check the ignitation (IGN must be HIGH)

Step 3: Check if the driver is seated and the driver's seat belt is unfastened.

- DRIV = 1, BELT_D̅ = 0

Step 4: Check if passenger is seated and passenger seat belt is unfastened.

- PASS = 1, BELT_P̅ = 0

Step 5: IF EITHER of the above conditions are TRUE and IGN = 1, then alarm activates to low (0)

# 3.2 Truth Table

| IGN | DRIV | PASS | BELT_D̅ | BELT_P̅ | ALARM |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |

3.3 Boolean Expressions (SOP)

- DRIV. ~PASS. ~BELT_D̅. ~BELT_P̅. IGN

- DRIV. ~PASS. ~ BELT_D̅. BELT_P̅. IGN

- DRIV. PASS. ~ BELT_D̅. ~BELT_P̅. IGN

- DRIV. PASS. ~BELT_D̅. BELT_P̅. IGN

- DRIV. PASS. BELT_D̅. ~BELT_P̅. IGN

- ALARM = DRIV. ~PASS. ~BELT_D̅. ~BELT_P̅. IGN + DRIV. ~PASS. ~ BELT_D̅. BELT_P̅. IGN + DRIV. PASS. ~ BELT_D̅. ~BELT_P̅. IGN + DRIV. PASS. ~BELT_D̅. BELT_P̅. IGN + DRIV. PASS. BELT_D̅. ~BELT_P̅. IGN

# 3.4 Flowchart

# 3.5 Pseudocode

START

GET DRIV, BELT_D̅, BELT_P̅, IGN

IF DRIV. ~PASS. ~BELT_D̅. ~BELT_P̅. IGN + DRIV. ~PASS. ~ BELT_D̅. BELT_P̅. IGN + DRIV. PASS. ~ BELT_D̅. ~BELT_P̅. IGN + DRIV. PASS. ~BELT_D̅. BELT_P̅. IGN + DRIV. PASS. BELT_D̅. ~BELT_P̅. IGN

THEN ALARM = 0 #ON

ELSE ALARM = 1 #OFF

# 3.6 BOOLEAN CIRCUIT

# Step 4: Test and Refine the Solution (Debug and Verify)

Test Case 1: Car not started

- IGN=0, DRIV=1, PASS=1, BELT_D̅=0, BELT_P̅=0
 Expected: ALARM=1 (alarm must stay OFF because IGN=0)

Test Case 2: Car started, no one seated

- IGN=1, DRIV=0, PASS=0, BELT_D̅=0, BELT_P̅=0
 Expected: ALARM=1 (no seat occupied)

Test Case 3: Driver seated, driver belt unfastened

- IGN=1, DRIV=1, PASS=0, BELT_D̅=0, BELT_P̅=1
 Expected: ALARM=0 (unsafe driver)

Test Case 4: Driver seated, driver belt fastened

- IGN=1, DRIV=1, PASS=0, BELT_D̅=1, BELT_P̅=0
  Expected: ALARM=1 (driver safe; passenger not seated)

Test Case 5: Passenger seated, passenger belt unfastened

- IGN=1, DRIV=0, PASS=1, BELT_D̅=1, BELT_P̅=0
  Expected: ALARM=0 (unsafe passenger)

Test Case 6: Passenger seated, passenger belt fastened

- IGN=1, DRIV=0, PASS=1, BELT_D̅=0, BELT_P̅=1
  Expected: ALARM=1 (passenger safe)

Test Case 7: Both seated, both belts unfastened

- IGN=1, DRIV=1, PASS=1, BELT_D̅=0, BELT_P̅=0
 Expected: ALARM=0 (unsafe)

Test Case 8: Both seated, both belts fastened

- IGN=1, DRIV=1, PASS=1, BELT_D̅=1, BELT_P̅=1
 Expected: ALARM=1 (safe)